# Pitch Speech — Real-Time Motor Stall Detection

≈ 2:45  ·  Toyota Innovation Challenge — Fault Prediction

Read top to bottom. [SLIDE n] = advance the slide.  Bold = emphasis.  [brackets] = stage action.

SLIDE 1 · Title

Unexpected machine downtime costs manufacturers millions a year. The earliest warning a motor gives before it fails is its own current — so we built a system that reads that current in real time and catches a stall the instant it happens.

SLIDE 2 · The trap

Here's what makes this hard. When a motor stalls, current spikes — but current also spikes every single time the motor starts: the inrush. A naive 'current-too-high' alarm would cry wolf on every power-on. The real signal isn't high current — it's high current that stays high. A stall is sustained; a start-up just decays. Telling those two apart is the heart of our project.

SLIDE 3 · How it works

Under the hood: we slice the current into short windows, pull out six features — level, slope, and whether it's still rising or holding — and feed a small decision tree. We chose a tree on purpose: it's interpretable, and tiny enough to run on a chip. And the algorithm itself discovered the key rule — that steadiness is what separates a real stall from a start-up.

SLIDE 4 · Live demo

[switch to the live dashboard / the bench]  Let me show you. Running — green. Watch when I power-cycle it — [power-cycle the motor] — the current spikes, but it stays green. No false alarm. Now I actually stall it — [hold the shaft] — RED, in under a tenth of a second. Release — back to green. Start-up ignored, real stall caught instantly.

SLIDE 5 · Results

On our collected data it cleanly separates the three states. Stall is perfectly classified — never missed — and a start-up is never predicted as a stall. That's the property that matters. The whole pipeline has eighteen automated tests and a physics simulator, so we validated it before we ever touched hardware.

SLIDE 6 · Edge ML

And it doesn't need a laptop. We compiled that same decision tree into C and flashed it onto the Arduino itself. The board runs the inference on-chip and lights this LED on a stall. [point to the board's LED] Pull the laptop, power it from a charger — it still works. Detection at the edge, on a microcontroller — exactly where a factory sensor lives.

SLIDE 7 · Scale

And the method isn't locked to our bench: the exact same feature code runs unchanged on Toyota's real eight-DOF arm telemetry, across the whole fleet — and it surfaces the most mechanically-stressed joints, the shoulder and elbow, exactly where the load is. With labeled fault data, the same approach scales to production predictive maintenance.

SLIDE 8 · Safety, Human, Close

It's human-centered: it catches failures before they cascade — protecting equipment, preventing unplanned line stops, and keeping workers out of harm's way. From a three-dollar motor to a robot fleet — same physics, same code. We turn a motor's current into an early-warning system. Thank you.

Timing (~2:45)

| S1 | S2 | S3 | S4 demo | S5 | S6 | S7 | S8 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0:15 | 0:30 | 0:30 | 0:45 | 0:20 | 0:20 | 0:20 | 0:15 |

Rubric coverage: Ideation → inrush-vs-stall insight (S2) · Execution → live demo + tests (S4/S5) · Safety & Human → S8 · Presentation → interpretable tree + clean visuals.

If asked (Q&A)

“Did you reuse the small-motor model on the arm?”  → No. Only the feature-extraction code transfers; each deployment trains its own model. We do not claim arm stall detection — that data is unlabeled.

“Why a decision tree, not a neural net?”  → Interpretable, fits on a chip, and it's already perfect on the critical class. We can show the exact learned rule.

“Different motor / generalization?”  → Recollect a few minutes, retrain in seconds; the features are threshold-free, and we train and infer on the same scale.

“Are the Toyota red cells faults?”  → No — highest mechanical-stress windows, not confirmed faults. A real jam would look the same and be caught the same way.

“How is this different from a current relay?”  → A relay only sees a threshold — it false-alarms on every start-up. We distinguish inrush from stall.